1.Problem Formulation

This section formulates a task-space trajectory tracking problem for a serial robotic manipulator subject to kinematic and actuator constraints. The nonlinear model predictive control (NMPC) framework considered in this work is applicable to general task-space trajectory tracking problems. Its advantage becomes particularly evident when the desired trajectory approaches the boundary of the robot’s reachable workspace, where joint constraints and nonlinear kinematics significantly restrict admissible motions. By explicitly considering future constraint activation, NMPC enables stable and feasible tracking in both nominal and boundary operating conditions.

1.1 Robotic System Description

The system considered in this work is a serial robotic manipulator with six revolute joints, resulting in six degrees of freedom. The configuration of the robot is described by the joint position vector

q∈R6

The robot operates in a fixed-base configuration. Each joint is subject to physical position and velocity limits, which must be satisfied during motion execution.

1.2 Kinematic Model

A kinematic model is adopted to describe the motion of the manipulator. This model captures the nonlinear relationship between joint motion and end-effector motion and is well suited for task-space trajectory tracking at moderate speeds.

- 1.2.1 Forward Kinematics

The task-space position of the end-effector is given by the nonlinear forward kinematics

y=f(q)

wherey∈Rmdenotes the Cartesian position of the end-effector, andf(·) represents the forward kinematic mapping.

- 1.2.2 Differential Kinematics

The relationship between joint velocities and task-space velocities is described by

y=J(q)q

where Jq is the manipulator Jacobian matrix. In this work, the Jacobian is not used directly for control computation. Instead, it provides a conceptual description of how joint-space motion affects task-space behavior. As the manipulator approaches workspace boundaries or kinematic singularities, the Jacobian may become ill-conditioned, which reduces the set of feasible joint velocity commands. This effect motivates the use of a predictive control framework that can anticipate constraint activation.

1.3 State and Control Variables

The system state is defined as the joint position vector,

xk=qk

and the control input is defined as the joint velocity,

uk=qk

This formulation allows joint position and velocity constraints to be enforced directly and enables the control inputs to be applied as joint velocity commands without requiring inverse kinematics.

1.4 Task Definition

- 1.4.1 Task-space Output

Although the system state is defined in joint space, the controlled output of interest is the end-effector position in task space. The task-space output is given by

yk=f(qk)

- 1.4.2 Reference Trajectory and Tracking Error

A desired task-space trajectory is specified as

yd,k

The task-space tracking error is defined as

ek=yk-yd,k

By defining the tracking error in task space, the control objective is directly aligned with the task specification.

1.5 Constraints

The manipulator is subject to joint-level constraints that ensure safe and feasible operation.

- 1.5.1 Joint Position Constraints

Joint position limits are imposed as

qmin≤qk≤qmax

- 1.5.2 Joint Velocity Constraints

Joint velocity limits are imposed as

qmin≤uk≤qmax

Workspace feasibility is handled implicitly through joint constraints and the nonlinear kinematic mapping. When the end-effector trajectory approaches the boundary of the reachable workspace, constraint activation may occur within the prediction horizon, even if the current state is feasible. This motivates the use of a predictive control formulation.

The task-space trajectory tracking problem is formulated as a finite-horizon optimal control problem. At each control step, a sequence of joint velocity inputs is computed to minimize the task-space tracking error while respecting joint position and velocity constraints.

The optimization problem is subject to the kinematic model, joint constraints, and the current joint configuration as the initial condition. This formulation serves as the basis for the NMPC design presented in the following section.

2. Nonlinear MPC Design and Solution

This section describes the NMPC controller implemented in this work. Based on the problem formulation in Section 3, the controller computes joint velocity commands to track a task-space trajectory under joint position and velocity constraints. The design choices presented below are closely aligned with the specific characteristics of the considered system and simulation setup.

2.1 Discrete-Time Prediction Model

The prediction model used in the NMPC is a discrete-time kinematic model with joint velocities as control inputs:

qk+1=qk+Δtuk

The first-order integrator matches the control interface of the simulated robot. In the considered setup, joint velocity commands are tracked by a low-level controller that compensates for robot dynamics. As a result, the closed-loop behavior observed by the NMPC can be approximated by a first-order integrator in joint space. This modeling choice is consistent with the hierarchical control structure of the system and avoids introducing unnecessary dynamic complexity into the NMPC formulation.

2.2 NMPC Optimization Problem

At each control step, the NMPC solves a finite-horizon optimization problem to compute a sequence of joint velocity inputs that minimize task-space tracking error while respecting joint constraints.

- 2.2.1 Cost function

The cost function penalizes task-space tracking error and joint velocity magnitude:

J=i=0N-1(ek+iТQek+i+uk+iТRuk+i)

where ek+i​ denotes the task-space tracking error at prediction step k+i, Q and R are positive semi-definite and positive definite weighting matrices, respectively.

The task-space error term ensures that the end-effector follows the desired Cartesian trajectory, while the control regularization term limits aggressive joint velocity commands and contributes to smooth motion.

- 2.2.2 Prediction horizon and control horizon

The prediction horizon N determines how far into the future the NMPC anticipates system behavior and constraint activation. A longer horizon allows the controller to better anticipate upcoming workspace limitations and joint constraint interactions, especially when the desired trajectory approaches the boundary of the reachable workspace. However, increasing the horizon length also raises computational complexity.

In this work, the prediction horizon is selected to provide sufficient look-ahead capability while maintaining real-time feasibility. The control horizon is set equal to the prediction horizon, meaning that a control input is optimized at every prediction step. This choice simplifies the formulation and is suitable for the considered kinematic control problem.

2.3 Receding Horizon Strategy

The NMPC operates in a receding horizon manner. At each sampling instant, the optimization problem is solved using the current joint configuration as the initial state, and only the first joint velocity command is applied. This closed-loop strategy allows the controller to continuously correct tracking errors and adjust control actions as constraints become active along the trajectory.

2.4 Numerical Solution

The optimization problem is nonlinear due to the forward kinematics in the task-space error term. A nonlinear solver is employed and warm-started using the shifted solution from the previous control step. The computational cost of the NMPC optimization is acceptable for offline trajectory generation and simulation-based validation.